ELC201A-Project-1

[1] Transistor Characterization:

- Perform DC sweep for Vout (from 0 to 3V) while Vin=2V and plot ID vs. Vout.

By sweeping Vout from 0 to 3 volts and observe the change of ID:

Vout vary from 0 to 3 volts, by step 0.1.

By using the Multimeter to measure the current of ID of the corresponding Vout.

The Results:

We used MATLAB to plot the result as shown in the figure.

From the curve shown we can notice that Vth =0.95 V

By Taking the slope of the curve from Vout = 0.95 Volt to 3.0 Volt to get ro.

Using the built-in function in MATLAB to get slope:

Therefore slope = 5.5021 μAV-1 .

Then ro=1slope=181.748 KΩ

Since  λ = 1ro Id=0.019 μV-1, where Id=288.8907 μA from the mean of the saturation region

Perform DC sweep for Vin (from 0 to 3V) while Vout=2V and plot ID vs. Vin.

Same as the above problem

By sweeping Vin from 0 to 3 volts and observe the change of ID:

Vin vary from 0 to 3 volts, by step 0.1.

By using the Multimeter to measure the current of ID of the corresponding Vin.

The Results:

By plotting the results:



To get gm:

We need to get gm from figure of  Id vs. Vin.

Since gm= ∂Id∂Vin=slope

Using MATLAB to get slope

Therefore gm=257.4625  μAV-1

To get μnCOX:

Since, Id= 12 μnCOXwl(Vgs - Vth)2, by substituting Id=288.8907 μA, Vgs=2 V. μnCOX= 1.0521* 10-4.

[2]  Single Stage Amplifier:

Dc Analysis

- V TH =0.95 V ,

- gm=2ID×Kn=4×10-3 .

3) Assume L=2u ,  W=300u ,  µn Cox =1.0521× 10-4  ID=100 UA.

4) Assume Saturated:

- ID=12 ×Kn ×(Vgs-Vth) 2.

- V gs =1.06257 volt.

- VDD-VD=ID ×R .

- VD > VG- V TH.

- -ID ×R+VDD>VG-VTH .

5) From high frequency

- R<10.61k Ω .

- vg < 2.8889
Let Vg=V dc =2volt.

- R=10k Ω .

6) From the low frequency:

- C=3.1uF .

Midband Gain

- Gainin db=20logVoutVin=20log21.4260=26.618822 db .

Low Frequency Analysis

- W low= 1C×1gm= 1C × 14×10-3=2π ×500 .

- c=3.1uF.

Simulation For AC Sweep

High Frequency Analysis

- Whigh= 1CL ×R=110-12×R1=15 M hz .

- R <10.61 kΩ .

Rin And Rout

- R in= 1gm=250  Ω .

- R out=10 KΩ  .

Transient analysis - Vin=A × sin(2πft) . - A= 10 mV.

- IF F=100 K hz.

- IF F=50 hz .

- IF F=150 M hz .